Phosphorus ( mg/L)

### Canada (CCME – Canadian Council of Ministers of the Environment)

The Canadian Water Quality Guidelines for the Protection of Aquatic Life don’t give a single hard threshold, but instead adopt the OECD trophic classification, based on total phosphorus (TP) in open-water samples:

- Oligotrophic target (pristine, coldwater lakes): ≤ .01mg/L

- General “acceptable” level for recreation / cottage use: ≤ .02 mg/L

- Mesotrophic (moderate nutrients, some algae growth):
≈ .01–.02 mg/L

- Warning zone: .02–0.1 mg/L → algae likely, declining quality

|  | Fraser 1 | Fraser 2 | Fraser 3 | Fraser 4 | Bell1 | Bell2 | Bell3 | Bell4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Aug 6, 2025. Micro b | <.02 | <.02 | <.02 | <.02 | <.02 | <.02 | <.02 | <.02 |

Ecoli (UFC/100ml)

- Must be 0 to be drinkable.  Otherwise, unsafe

- < 200 is acceptable for swimming

|  | Fraser 1 | Fraser 2 | Fraser 3 | Fraser 4 | Bell 1 | Bell 2 | Bell 3 | Bell 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Aug 6, 2025. Micro b | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |